f                           										Comlock.wpd

HOW TO OPEN A 1200 / 1220 SERIES SAFE

- Take the keys and insert one into the lock,

- Now set the combination dial at 0 against the red marker at 12 o’ clock,

- Turn the dial to the right (clockwise) 4 full revolutions ending back on the 0,

- Look on the combination card for the first combination number,

- Still turning in the same direction to the right (clockwise) go to the first combination number and stop accurately on the number ensuring you do not go past the number,

- Again look at the combination card for the second number,

- Now turn the dial to the left (anti-clockwise) until you get to the second number,

- Still turning the dial in the same direction to the left (anti-clockwise) complete 2 full revolutions ending back on the second number and again ensuring you do not go past the number on the final revolution,

- Again look at the combination card for the third number,

- Now turn the dial to the right (clockwise) until you get to the third number,

- Still turning the dial in the same direction to the right (clockwise) complete 1 full revolution ending back on the third number and again ensuring you do not go past the number,

- Again look at the combination card for the forth and final number,

- Now turn the dial to the left (anti-clockwise) until you get to the forth number and  again ensuring you do not go past the number,

- Turn the key to the right (clockwise) and open the door.

# TO LOCK THE SAFE

- Close the door,

- Turn the key to the left (anti-clockwise),

- And remove the key from the lock,

- Now turn the dial to the left (anti-clockwise) two revolutions,

- Your safe is now locked and secure.